APPENDIX 1. SPECIFICATION OF THE EQUIPMENT

9 5/8” x 7” (5 1/2”) x 2 7/8” 3000 PSI

| WELLHEAD & X-MAS TREE ASSEMBLY, 9 5/8” x 7” (5 1/2”) x 2 7/8” 3000 PSI W.P. PSL 2, PR 2, TEMPERATURE CLASS P+U, MATERIAL CLASS AA Manufactured in accordance with API 6A; Twenty-First (21th) Edition, November 2018 API Spec Q1 |  |  |  |
| --- | --- | --- | --- |
| Detailed specification of equipment and parts listed by section below: |  |  |  |
| Section A.1: For each (individual) X-mass tree |  |  |  |
| Item | Qty | Casing Head Assembly, consisting of: |
|  | 1 | Landing base mounting on BTM 13 3/8” OD Conductor; TOP saddle groove for support landing ring |
|  | 1 | Casing head housing, 9 5/8” Slip and Lock BTM x 11” 3000psi WP top flange. Lock screws to hold the BOP test plug & Wear Bushing in place casing heads BTM support landing ring (RH Stub Acme thread) With 2 x 2 1/16” 3000 psi WP studded side outlets, completed with stud and nuts. |
|  | 1 | Casing Hanger (slip) & seal assembly, 11” x 5 1/2” OD Casing. |
|  | 1 | Expanding Gate Valve, 2 1/16” 3000 psi WP manual operated using hand-wheel, flanged ends, full bore design |
|  | 2 | Companion flange 2 1/16” x 2” LP 3000psi |
|  | 2 | Bull Plug, 2” LP x 1/2” NPT.Box |
|  | 2 | Needle valves, angle, 1/2” NPT male x female 3000 psi WP. |
|  | 1 | Pressure gauge for range 4000 psi, 1/2“ NPT, 4 1/2” or 6 ¼” face |
|  | 1 | Ring Gasket API R53, Cadmium plated, for 11” x 3000psi flange |
|  | 3 | Ring gasket API R24, cadmium plated for 2 1/16” x 3000psi flange |
|  | 1 | Set of 16 Studs complete with 2 Nuts each 1 3/8” X 10” Zinc plated suitable for top flange of Casing Head spool |
|  | 1 | Set of 8 Studs with 2 Nuts each 7/8” x 6 1/2” Zinc plated suitable for 2 1/16” flange. |
| Section A.2: For complete delivery X-mass trees (for YY X-mass trees) |  |  |  |
| - 1 | 6 | Casing Hanger (slip) & seal assembly, 11” x 7” OD Casing. |
| Section B.1: For each (individual) X-mass tree |  |  |  |
| Item | Qty | Tubing Head Spool Assembly, consisting of: |
|  | 1 | Tubing Head Spool, 11” 3000 psi WP bottom flange x 7 1/16” 3000 psi WP top flange, with 2 x 2 1/16” 3000 psi WP studded side outlets completed with studs and nuts |
|  | 1 | Seal assembly, 11” x 5 1/2” OD Casing |
|  | 2 | Expanding Gate Valves, 2 1/16” 3000 psi WP manual operated using hand-wheel, flanged ends, Full bore design |
|  | 4 | Ring gasket API R24, cadmium plated for 2 1/16” x 3000psi flange |
|  | 2 | Set of 8 Studs with 2 Nuts each 7/8” x 6 1/2” Zinc plated suitable for 2 1/16” flange. |
|  | 2 | Companion flange, size 2 1/16” 3000 psi WP x 2” LP thread |
|  | 1 | Tubing Hanger Assembly, 2 7/8” API EUE top x bottom thread Inside thread 2 31/64” – 4 BPV 90° LH 2B (for BPV test plug) Including 5 sets of sealing elements. Note: - Max OD 178 mm - ID 61,5 mm - Along with the offer, submit complete technical documentation and drawings to view. - The customer reserves the right to request corrections of the construction in accordance with his need |
|  | 1 | Ring Gasket API R45 for 7 1/16” 3000psi Cadmium plated |
|  | 1 | Needle valves, angle, ½” NPT male x female 3000 psi WP. |
|  | 1 | Needle valve, straight, ½” NPT male x female 3000 psi WP. |
|  | 2 | Pressure gauge for range 4000 psi with needle valve 1/2“ NPT , 4 1/2” or 6 ¼” face |
| Section B.2: For complete delivery X-mass trees (for YY X-mass trees) |  |  |  |
|  | 6 | Seal assembly, 11” x 7” OD Casing |
| Section C: For each (individual) X-mass tree |  |  |  |
| Item | Qty | X-Mas tree Assembly, consisting of: |
|  | 1 | Tubing Adapter Assembly, 7 1/16” 3000 psi WP flanged bottom x 2 9/16” 3000 psi WP studded top completed with studs and nuts |
|  | 1 | ESP adapter 7 1/16” 3000 psi WP flanged bottom x 2 9/16” 3000 psi WP top flange; Thread port 2” API.LP. Sealing head for ESP Cable (penetrator). Provide sealing elements for ESP cable Φ11 and Φ13. Note: - Along with the offer, submit complete technical documentation and drawings to view. - The customer reserves the right to request corrections of the construction in accordance with his need. |
|  | 3 | Expanding Gate Valves, 2 9/16” 3000 psi WP manual operated using handwheel, flanged ends, Full bore design |
|  | 5 | Ring Gasket API R27 for 2 9/16” 3000psi, Cadmium plated. |
|  | 1 | Cross, studded, four way, 2 9/16” x 2 1/16” 3000 psi WP completed with studs and nuts |
|  | 2 | Expanding Gate Valves, 2 1/16” 3000 psi WP manual operated using handwheel, flanged ends, Full bore design |
|  | 4 | Ring gasket API R24, cadmium plated for 2 1/16” x 3000psi flange |
|  | 1 | Companion flange, size 2 1/16” 3000 psi WP x 2” LP thread. |
|  | 1 | Positive chokes with flanged ends 2 1/16” x 3000psi 2,4 mm; 2,6 mm; 2,8 mm; 3,0 mm; 3,2 mm; 3,4 mm; 3,6mm |
|  | 1 | Bull Plug, 2” LP x ½” NPT female thread. |
|  | 1 | X-mas tree cap assembly, 2 9/16” 3000 psi WP flanged bottom with 2 7/8” OD EUE pickup thread. |
|  | 1 | Set of 12 Studs with 2 Nuts each 1 1/8” x 8 1/2” Zinc plated suitable for 7 1/16” flange. |
|  | 2 | Set of 8 Studs with 2 Nuts each 1” x 7” Zinc plated suitable for 2 9/16” flange. |
|  | 2 | Set of 8 Studs with 2 Nuts each 7/8” x 6 1/2” Zinc plated suitable for 2 1/16” flange. |
|  | 1 | Needle valves, angle, ½” NPT male x female 3000 psi WP. |
|  | 1 | Needle valve, straight, ½” NPT male x female 3000 psi WP. |
|  | 2 | Pressure gauge for range 4000 psi with needle valve 1/2“ NPT, 4 1/2” or 6 ¼” face |
| Section D: For complete delivery X-mass trees (for YY X-mass trees) |  |  |  |
| Item | Qty | BOP test plug & Wear Bushing for Casing Head: |
|  | 6 | BOP test plug & Wear Bushing for 11” Tool To Install / Remove The Wear Bushing or Combined Tool. Every set equipped with 5 sets of sealing elements. Note: The drill pipe 5“ G-105/S-135 19,5 # connection 4 ½” IF can be suspended below the test plug, thus eliminating the need to trip the whole pipe out of the hole. |
| Section E: For complete delivery X-mass trees (for YY X-mass trees) |  |  |  |
|  | 3 | BOP test plug & Wear Bushing for 7 1/16” Tool To Install / Remove The Wear Bushing or Combined Tool. Every set equipped with 5 sets of sealing elements. Note: The drill pipe 2 7/8“ G-105 10,40# connection 2 7/8” IF (NC31) can be suspended below the test plug, thus eliminating the need to trip the whole pipe out of the hole. |
| Section F: For complete delivery X-mass trees (for YY X-mass trees) |  |  |  |
| Item | Qty | Test plug & Back pressure valve |
|  | 6 | Solid Test Plug for tubing hanger Including 3 sets of sealing elements. |
|  | 6 | Type “H” BPV - Back pressure valves for tubing hanger Including: - 5 sets of sealing elements. - 1 redress kit for BPV Back pressure valves for tubing hanger Type “H” |
|  | 6 | Type “H” TWCV - Back pressure valves for tubing hanger Including: - 5 sets of sealing elements. - 1 redress kit for TWCV Back pressure valves for tubing hanger Type “H” |
|  | 6 | Running tool for Back pressure valves for tubing hanger |
|  | 6 | Retrieving tool for Back pressure valves for tubing hanger |
|  | 6 | Solid Test Plug – Valve removal plug |
| Section G: For complete delivery X-mass trees (for YY X-mass trees) |  |  |  |
| Item | Qty | Lubricator |
| 1. | 1 | The lubricator - tool for installing or retrieving BPV plug type H back - pressure valve under pressure in the well. Including friction wrench 2 pcs Note: - The offered set must contain all the parts and tools necessary for safe work with the equipment - Specify the content of the offered set (PN and full name) - Along with the offer, submit complete technical documentation and work instructions for review |
| 2. | 1 | The lubricator - tool for installing or retrieving valve removal plug – solid test plug under pressure in the well. Including friction wrench 2 pcs Note: - The offered set must contain all the parts and tools necessary for safe work with the equipment - Specify the content of the offered set (PN and full name) - Along with the offer, submit complete technical documentation and work instructions for review |
| Section H: For each (individual) X-mass tree |  |  |  |
| Item | Qty | Spare parts |
|  | 4 | Expanding Gate Valves, 2 1/16” 3000 psi WP manual operated using handwheel, flanged ends, Full bore design |
|  | 4 | Expanding Gate Valves, 2 9/16” 3000 psi WP manual operated using handwheel, flanged ends, Full bore design |
|  | 10 kg | Grease for Body valve |
|  | 1 Pcs | Pressure gauge for range 4000 psi with needle valve 1/2“ NPT |
|  | 1 Pcs | Needle valves, angle, ½” NPT male x female 3000 psi WP. |
|  | 1 Pcs | Needle valve, straight, ½” NPT male x female 3000 psi WP. |
| Section I: For complete delivery X-mass trees (for YY X-mass trees) |  |  |  |
| Item | Qty | Spare parts for commissioning and maintenance |
|  | 5 sets | Casing Hanger (slip) & seal assembly, 11” x 5 1/2” OD Casing. |
|  | 6 Sets | Seal assembly, 11” Casing Head spool x 7” OD Casing |
|  | 6 Sets | Seal assembly, 11” Casing Head spool x 5 1/2” OD Casing |
|  | 6 Sets | Seal assembly, 11” Tubing Head spool x 7” OD Casing |
|  | 6 Sets | Seal assembly, 11” Tubing Head spool x 5 1/2” OD Casing |
|  | 6 Sets | Sets of sealing elements BOP test plug & Wear Bushing for 11” |
|  | 6 Sets | Sets of sealing elements BOP test plug & Wear Bushing for 7 1/16” |
|  | 6 Sets | Set top and Bottom Sealing elements for tubing hanger |
|  | 6 Sets | Set Lockscrews, Flange 11” Set including: Lockscrew pin, Gland nut, packing (sealing elements) |
|  | 6 Sets | Set Lockscrews, Flange 7 1/16” Set including: Lockscrew pin, Gland nut, packing (sealing elements) |
|  | 6 | Injection Gun for plastic packing seals (activating BTM “P” seals) |
|  | 6 | Plastic packing seals (activating BTM “P” seals) |
|  | 6 | Set of sealing elements for sealing the zone BTM "Casing head housing" / Casing 9 5/8" |
|  | 6 | A set of all necessary crossover coupling and quick couplings necessary for connection and joint testing and other necessary equipment as recommended by the manufacturer) |
|  | / | And other parts and equipment as recommended by the manufacturer that are necessary for the installation and commissioning of the specified equipment |