Прилози /посебни технички захтеви (дозвољена одступања, специфична паковања): / Attachmenrts /special technical requirements (allowed deviations, specific packaging):

Ponuđač treba da dostavi ponudu prema specifikaciji koja je sastavni deo ovog tehničkog zadatka

Ponuđač mora da prihvati odgovornost za eventualno isporučenu lošiju robu od ugovorene tj. specificirane

Ponuđač je u obavezi da isporuči pasoš opreme sa originalnim serijskim brojem

Nadzemna merno regulaciona - elektro oprema mora da bude u EX izvedbi i dadobavljač je dužan da kod akreditacionog tela urade reatset u skaldu sa zakonima Republike Srbije

U slučaju da kupac ima neke nedoumice vezane za pripremu, montažu i ugradnju kupljene opreme, dobavljač je dužan da elektronskim putem (putem maila ili video konferencije) u roku od 2 kalendarska dana od dana dostavljanja informacije dostavi preporuke i uptstvo za pripremu montažu i ugradnju opreme. Kontakt osoba Kompanija NIS a.d.: Miloš Aleksić milos.aleksic@nis.eu, tel. +381648884719. Ugovorom precizno definisati tehničko lice ponuđača za kontakt. U slučaju potereb za defektažom dobavljač je dužan da obezbedi prisustvo stručnog lica na lokaciji naručioca u roku od 5 kalendarskih dana. U slučaju problema sa softwareom regulatora, dobavljač mora obezbediti neophodnu podršku za otklanjanje problema.

Tehnički zahtevi za automatski kontroler ciklusa sa solarnim panelom:

• Temperatura rada: - 30 do + 70 ºC

• Snaga (Power eng.): 6 VDC standardne baterije

• Ulazni napon baterije (Battery input voltage.): 5.5-7 VDC

• Potrošnja (Consumption eng.): 10-380 mW

• Solarni sistem (Solar system.): 6 VDC, 3 Watt panel sa punjivom baterijom 6 VDC

• Displej: 4 linije – 80 karaktera

• Električna konekcija: 1-3 konekcije

• Pneumatske konekcije: 1-3 konekcije (Izvor gasa: ¼-18NPT 316SS, 0-90 PSIG (suvi gas), Ventil A: ¼-18NPT 316SS, Ventil B opciono: ¼-18NPT 316SS)

• Operacije: Klip lift operacije, periodične operacije, operacije diferencijalnog pritiska

Rok isporuke: Dobavljač mora isporučiti opremu na osnovu ovog tehničkog zadatka u roku od 90 dana od potpisivanja narudžbenice"

"Ponuđač treba da dostavi svu dokumentaciju o opremi koju isporučuje:

- Uputstvo za održavanje, skladištenje, montažu i eksploataciju.

- Tehničke crteže sa dimenzijama.

- Pismenu potvrdu overenu od dobavljača da je ponuđena oprema orginalna, nekorišćena i nova."

The Bidder should submit its Bid pursuant to the technical specification which is an integral part of this technical assignment.

The Bidder must accept responsibility for any delivered goods that are inferior to the contracted one, i.e. specified one.

The Bidder shall deliver the equipment passport with the original manufacturer's serial number.

The aboveground measuring and regulation -  electrical equipment must be Ex-proof and the Supplier is obliged to perform the recertification (re-attestation) in accordance with the laws of the Republic of Serbia at the accreditation body

In case the Client has some uncertainties related to the preparation, assembly and installation of the purchased equipment, the Supplier must submit recommendations and instructions for the preparation of the assembly and installation of the equipmentn in an electronic way (by e-mail or video conference) within 2 calendar days from the date of delivery of the information. Contact person in NIS j.s.c. Company: Milos Aleksic, milos.aleksic@nis.eu, tel. +381648884719. The contract should precisely define who is the Supplier's technical person for contact. In case of needs for the troubleshooting, the Supplier is obliged to ensure the presence of an expert person at the Client's location within 5 calendar days. In the event of a problem with the controller's software, the Supplier must provide the necessary support to eliminate the problem.

Technical specification for cycle controller with solar panel:

• Operating temperature: from - 30 to + 70 ºC

• Power: 6 VDC, standard battery

• Battery input voltage: from 5.5 to 7 VDC

• Consumption: from 10 to 380 mW

• Solar system: 6 VDC, 3 Watt panel with rechargeable battery of 6 VDC

• Display: 4 lines, 80 characters

• Electrical connections: from 1 connection to 3 connections

• Pneumatic connections: from 1 connection to 3 connections (gas source: ¼-18NPT 316SS, 0-90 PSIG (dry gas), valve A: ¼-18NPT 316SS, valve B optionally: ¼-18NPT 316SS)

• Operation: plunger lift operation, intermittent operations, differential pressure operations

Deadline for the delivery: The Bidder shall deliver the equipment based on this Technical Assignment within 90 days from the order signing date.

The Bidder should submit all the documentation on the equipment which will be delivered:

- Instructions for maintenance, storage, assembly and exploitation.

- Technical drawings with dimensions.

- Written confirmation which has been certified by the Supplier and which is used to prove that the proposed equipment is original, unused and brand-new."