APPENDIX 1. SPECIFICATION OF THE EQUIPMENT

| All equipment must be manufactured in accordance with: API 11D1; Fourth (4th) Edition, April 2022, V3, API Spec Q1 – for packers, redress kits for packers and pump out plugs API 14L, Third (3rd) Edition, June 2020, V2, API Spec Q1 - for landing nipples API 19AC, First (1st) Edition, September 2016, API Spec Q1 – for sliding sleeves, expansion joints and redress kits API Spec 5B, Sixteenth (16th) Edition, December 2017, API Spec Q1 – for every item in the Appendix, except redress kits |  |  |  |
| --- | --- | --- | --- |
| Detailed specification of equipment and parts listed by section below: |  |  |  |
| Section A.1 – Standard API 11D1; Fourth (4th) Edition, April 2022, V3, API Spec Q1 |  |  |  |
| Item | Required quantities | Optional quantities | Hydraulic production packers and redress kits |
|  | 5 | 2 | BRAND NEW HYDRAULIC SET RETRIEVABLE PACKER (5 1/2", 17 lb/ft, 2 7/8" EU, 325 F, 7500 PSI, L-80) with a minimum temperature rating of packing elements of 163 °C (325 °F) for casing OD=5 1/2", 17 lb/ft, K-55. Differential pressure on packer to be 7.500 psi. The packer should be activated in a single run by applying pressure and deactivated by tension, with no mandrel moving while setting, with triple sealing elements, compatible with TCP application, for inclination higher than 60 degrees, DLS 6/30m. |
|  | 15 | 4 | Redress kit per 1 hydraulic packer (for item 1), with packing elements for temperature 325 F. Note: Repair kit must contain all the removable parts which ensure undisturbed operation / complete service of the equipment (sealing elements, springs, screws, shear pins...). Confirmation that the sealing element are not older than six months. The delivery every sets in packing box. The customer reserves the right to check the completeness of the offered parts (sets) after reviewing the submitted technical documentation and requests the completion of sets in accordance with his needs. |
| Section A.2 – Standard API 14L, Third (3rd) Edition, June 2020, V2, API Spec Q1 |  |  |  |
| Item | Required quantities | Optional quantities | Lock mandrels, standing valves, landing nipples and redress kits |
|  | 2 | 0 | Brand new XN Landing Nipple NO-GO 2 3/8", max OD 3,668", Seal (Packing) Bore 1,875", with bottom NO-GO ID 1,791", L-80 material, 2 7/8"x6,5 lb/ft EU Box x Pin, 325 F, 10.000 psi, Otis type |
|  | 13 | 4 | Brand new XN Landing Nipple NO-GO 2 7/8" x 6,5 lb/ft, max OD 3,668", Seal (Packing) Bore 2,313", with bottom NO-GO ID 2,205", L-80 material, EU Box x Pin, 325 F, 10.000 psi, Otis type |
| Section A.3 – Standard API 19AC, First (1st) Edition, September 2016, API Spec Q1 |  |  |  |
| Item | Required quantities | Optional quantities | SSD Sliding sleeves |
|  | 13 | 4 | Brand new SSD Sliding Sleeve open up for tubing OD 2 7/8" x 6,5 lb/ft , Seal bore ID 2,312", Max OD 3,750, EUE Box x Pin, material L-80, 325 F, 10.000 PSI. XO shift down to open with shifting Otis tool “B” type. |
|  | 39 | 13 | Redress kit for Sliding Sleeve for 2 7/8" x 6,5 lb/ft tubing, 325 F, 10.000 PSI (for item 1) Note: Repair kit must contain all the removable parts which ensure undisturbed operation / complete service of the equipment (sealing elements, springs, screws, shear pins...). Confirmation that the sealing element are not older than six months. The delivery every sets in packing box. The customer reserves the right to check the completeness of the offered parts (sets) after reviewing the submitted technical documentation and requests the completion of sets in accordance with his needs. |
| Section B.1 – Standard API 19AC, First (1st) Edition, September 2016, API Spec Q1 |  |  |  |
| Item | Required quantities | Optional quantities | Expansion joints |
|  | 4 | 1 | Splined expansion joint OD=2 7/8” 6,5 lb/ft EU N80 with minimal stroke of 10ft, to be delivered without pins installed, full stroke mobile ready, for temperature 325 F, differential pressure 7.500 psi. |
|  | 4 | 1 | Redress kit for Splined expansion joint OD=2 7/8” 6,5 lb/ft EU N80 with minimal stroke of 10ft, for temperature 325 F, differential pressure 7.500 psi. (for item 1) Note: Repair kit must contain all the removable parts which ensure undisturbed operation / complete service of the equipment (sealing elements, springs, screws, shear pins...). Confirmation that the sealing element are not older than six months. The delivery every sets in packing box. The customer reserves the right to check the completeness of the offered parts (sets) after reviewing the submitted technical documentation and requests the completion of sets in accordance with his needs. |

| Section B.2 – Standard API 11D1; Fourth (4th) Edition, April 2022, V3, API Spec Q1 |  |  |  |
| --- | --- | --- | --- |
| Item | Required quantities | Optional quantities | Pump out plugs |
|  | 5 | 2 | Pump out plugs, Solid type (with burst disc) or Ball seated type OD 2 7/8”, rated for at least 5.000 psi differential pressure to burst, with integrated Wireline re-entry guide (sufficient maximum pressure to ensure successful packer activation procedure) |
|  | 5 | 2 | Redress kit for Pump out plugs, Solid type (with burst disc) or Ball seated type OD 2 7/8”, rated for at least 5.000 psi differential pressure to burst, with integrated Wireline re-entry guide (for item 1) Note: Repair kit must contain all the removable parts which ensure undisturbed operation / complete service of the equipment (sealing elements, springs, screws, shear pins...). Confirmation that the sealing element are not older than six months. The delivery every sets in packing box. The customer reserves the right to check the completeness of the offered parts (sets) after reviewing the submitted technical documentation and requests the completion of sets in accordance with his needs. |